Arduino Water Level Indicator – Lab Report

Submitted by:

Name: Fatema Tahsin Anamika

ID: 2021-3-60-170

Submitted to:

Dr. Raihan Ul Islam

Associate Professor

Introduction

This experiment explores the design and implementation of a water level indicator system using Arduino. The system utilizes an analog water level sensor and three LEDs to visually represent low, medium, and high water levels. Real-time sensor readings are monitored through the Arduino Serial Monitor, facilitating calibration and analysis.

Components and Circuit Connections

Components List

- Arduino Uno board

- Analog water level sensor

- Red, yellow, and green LEDs

- 3 × 220Ω resistors (for LEDs)

- Jumper wires

- Breadboard

Pin Mapping and Circuit Diagram

| Arduino Pin | Connected To | Purpose |
| --- | --- | --- |
| A0 | Sensor signal | Reads analog water level |
| 2 | Red LED (anode) | High water level indicator |
| 3 | Yellow LED (anode) | Medium water level indicator |
| 4 | Green LED (anode) | Low water level indicator |
| GND | LED cathodes, sensor GND | Common ground |
| 5V | Sensor VCC | Sensor power supply |

Connection Summary:

- Sensor signal connects to A0.

- Each LED’s anode connects to its assigned digital pin via a resistor; cathodes go to GND.

- Sensor powered by 5V and GND.

Detailed Code Analysis

const int sensorPin = A0;
const int redLED = 2;
const int yellowLED = 3;
const int greenLED = 4;

void setup() {
  pinMode(redLED, OUTPUT);
  pinMode(yellowLED, OUTPUT);
  pinMode(greenLED, OUTPUT);
  Serial.begin(9600);
}

void loop() {
  int sensorValue = analogRead(sensorPin);
  Serial.println(sensorValue);

  if (sensorValue < 300) {
    digitalWrite(redLED, LOW);
    digitalWrite(yellowLED, LOW);
    digitalWrite(greenLED, HIGH);
  } else if (sensorValue < 600) {
    digitalWrite(redLED, LOW);
    digitalWrite(yellowLED, HIGH);
    digitalWrite(greenLED, LOW);
  } else {
    digitalWrite(redLED, HIGH);
    digitalWrite(yellowLED, LOW);
    digitalWrite(greenLED, LOW);
  }

  delay(500);
}

Pin and Variable Declarations

- sensorPin (A0): Reads the analog voltage from the water level sensor, which varies with water level.

- redLED, yellowLED, greenLED: Digital output pins for the respective LEDs.

Setup Function

- Configures the three LED pins as outputs.

- Initializes serial communication at 9600 baud for real-time monitoring.

Main Loop Function

- Sensor Reading:
analogRead(sensorPin) captures the instantaneous water level as a value between 0 and 1023.

- Serial Output:
Serial.println(sensorValue); prints the sensor value to the Serial Monitor, aiding calibration and troubleshooting.

- Decision Logic:

  - Low (<300): Green LED ON (tank almost empty).

  - Medium (300–599): Yellow LED ON (tank partially filled).

  - High (≥600): Red LED ON (tank nearly full/full).

- LED Control:
Only one LED is ON at a time, corresponding to the detected water level.

- Delay:
delay(500); introduces a half-second pause for stability and readability.

. System Operation

- Sensor Detection:
The analog sensor outputs a voltage proportional to the water level. As water rises, the voltage (and the analog value) increases.

- Threshold Comparison:
The code compares the sensor value to two thresholds (300 and 600) to determine the water level category.

- Visual Feedback:
The corresponding LED is illuminated to indicate the current water status.

- Serial Monitoring:
Sensor values are sent to the Serial Monitor every 0.5 seconds for real-time observation.

Output Analysis

Serial Monitor Output

Below is the actual Serial Monitor output from the experiment:

Data Interpretation

- Initial Readings (0-299):
The sensor is dry or not in contact with water. The green LED is ON, indicating a low water level.

- Gradual Increase (50, 318, 393, ...):
As water is introduced, the sensor value rises, showing the system’s sensitivity and responsiveness.

- Medium Range (300–599):
Values in this range indicate a moderate water level, with the yellow LED ON.

- High Range (600 or more):
When readings reach or exceed 600, the red LED lights up, signaling a high water level.

System Responsiveness

- The transition from 0 to higher values demonstrates the sensor’s ability to detect even small amounts of water.

- The steady increase and intermediate values confirm the system can track gradual changes, not just abrupt transitions.

- The highest readings suggest the sensor is fully submerged or at its maximum detection point.

Discussion

- Accuracy:
The system reliably distinguishes between low, medium, and high water levels using well-defined thresholds.

- Real-Time Feedback:
Both the LED indicators and Serial Monitor provide immediate feedback, making the system practical for real-world applications.

- Calibration:
The Serial Monitor output is essential for adjusting threshold values to match the sensor and environmental conditions.

- Expandability:
The code and hardware setup can be easily expanded to include alarms, displays, or wireless notifications.

Conclusion

The Arduino-based water level indicator successfully monitors and displays water levels using analog sensing and threshold logic. The combination of clear visual indicators and real-time serial output makes it suitable for applications such as water tank monitoring, flood alerts, and irrigation systems.